Flip-Flops

- A flip-flop is a basic memory element in digital electronics.

- It can store 1 bit of data (either 0 or 1).

- Unlike combinational circuits, flip-flops are sequential circuits because their output depends not only on the current input but also on the previous state (due to memory).

- Flip-flops are built using logic gates (usually NAND or NOR gates).

- They are the building blocks for registers, counters, memory devices, and sequential logic circuits.

## Basic Characteristics of Flip-Flops

- Storage capacity: 1 bit

- Triggering: They change state on a clock pulse (edge-triggered or level-triggered). Triggering is the way a flip-flop decides when to store or change data, based on the clock pulse

- Inputs & Outputs:

  - Inputs: control what data is stored

  - Outputs: Q (current state) and Q̅ (complement of Q)

### Level Triggering

- The flip-flop responds as long as the clock pulse is HIGH (1) or LOW (0).

- Types:

  - Positive level triggering → Flip-flop works when clock is HIGH (1).

  - Negative level triggering → Flip-flop works when clock is LOW (0).

- Problem: If the input changes during the active level of the clock, the output may change multiple times → called race condition.

### 2. Edge Triggering

- The flip-flop responds only at the instant when the clocks signal changes.

- Types:

  - Positive edge triggering (↑) → Works only when clock changes from 0 → 1.

  - Negative edge triggering (↓) → Works only when clock changes from 1 → 0.

- Advantage: More reliable, avoids multiple changes because it reacts only at the moment of transition.

Types of flip-flops

SR Flip-flops

An SR flip-flop (Set-Reset flip-flop) is the simplest type of sequential logic circuit that can store 1 bit of information.

- It is called a bi-stable device because it has two stable states:

  - Set (Q = 1)

  - Reset (Q = 0)

- The output depends on both the current inputs and the previous output (feedback mechanism).

## Inputs and Outputs

- Inputs:

  - S (Set) → used to set the output to 1.

  - R (Reset) → used to reset the output to 0.

- Outputs:

  - Q → the current state.

  - Q̅ → the complement of Q.

## Logic Symbol

The SR flip-flop is generally represented with two inputs (S and R) and two outputs (Q and Q̅).

## Truth Table (NOR-based SR Flip-Flop)

| S | R | Q (Next State) | Description |
| --- | --- | --- | --- |
| 0 | 0 | No Change | Holds previous state |
| 0 | 1 | 0 | Reset state |
| 1 | 0 | 1 | Set state |
| 1 | 1 | Invalid | Not allowed (both 1) |

Invalid Condition: When both S=1 and R=1, both Q and Q̅ become 0 (which violates complement rule).

Characteristic Equation

The characteristic equation of SR flip-flop is:

This shows the next state of the flip-flop depends on Set input, Reset input, and the current state Q.

## Applications

- 1-bit memory cell (storing a single bit).

- Used in control circuits like registers, counters, and finite state machines.

- Basis for designing other flip-flops (JK, D, T).

## JK Flip-flops

A JK flip-flop is a type of sequential logic circuit.
It is an improved version of the SR flip-flop, which had an “invalid” condition (when both S=1 and R=1).
The JK flip-flop removes this problem and is widely used in digital electronics.

- Inputs: J, K, and Clock (sometimes Preset & Clear also).

- Outputs: Q (normal output) and Q′ (complement of Q).

- The JK flip-flop operates on the edge of the clock pulse (either rising or falling, depending on design).

- J → Set input (like S in SR flip-flop).

- K → Reset input (like R in SR flip-flop).

## 3. Logic Symbol

_________

J --|       Q |---> Output

K --| JK FF   |

Clk--|       Q'|---> Complement

|_________|

## 4. Truth Table of JK Flip-Flop

| J | K | Q(next) | Description |
| --- | --- | --- | --- |
| 0 | 0 | Q(prev) | No change (Hold state) |
| 0 | 1 | 0 | Reset (Clear) |
| 1 | 0 | 1 | Set |
| 1 | 1 | Q′ | Toggle (Complement) |

- When J=0, K=0 → No Change
Output remains same as previous state.
(Useful for memory storage).

- When J=0, K=1 → Reset
Output Q becomes 0, irrespective of previous state.

- When J=1, K=0 → Set
Output Q becomes 1, irrespective of previous state.

- When J=1, K=1 → Toggle
Output switches to its opposite state.
If Q=0 → Q=1, and if Q=1 → Q=0.
(This property is very useful in counters).

## Characteristic Equation

The equation defining the next state of JK flip-flop:

This means:

- If J=1 and Q=0 → Set Q to 1.

- If K=1 and Q=1 → Reset Q to 0.

- Otherwise hold.

## Types of JK Flip-Flops

- Level Triggered JK Flip-Flop
Works when clock is high or low continuously.
(Not preferred due to “racing” problem).

- Edge Triggered JK Flip-Flop
Works only on rising edge (↑) or falling edge (↓) of clock pulse.
This avoids instability.

- Master-Slave JK Flip-Flop

  - Consists of two JK flip-flops in series.

  - The first one (Master) responds to the leading edge of clock,
while the second one (Slave) responds to the trailing edge.

  - Prevents race-around condition.

## Race-Around Condition

- Problem occurs when J=K=1 and clock pulse is high for a long duration.

- Output toggles continuously during the high clock → undefined behavior.

- Solution: Use edge triggering or Master-Slave JK Flip-Flop.

## Applications of JK Flip-Flops

- Counters (up/down counters, ring counters).

- Shift Registers.

- Frequency division (toggle mode divides frequency by 2).

- Memory storage (to store 1-bit of data).

- Control circuits in computers and digital systems.

T flip-flops

Introduction

- A T Flip-Flop (Toggle Flip-Flop) is a sequential logic circuit.

- It is a modified form of JK Flip-Flop or D Flip-Flop.

- The name T stands for Toggle, because its main feature is switching (toggling) the output between 0 and 1 on each clock pulse when T=1.

Basic Concept

- Inputs: T (Toggle), Clock (and sometimes Preset & Clear).

- Outputs: Q and Q′.

- Operation principle:

  - When T=0 → Output holds the previous state.

  - When T=1 → Output toggles (switches to the opposite state).

## Logic Symbol

## Truth Table of T Flip-Flop

| T | Q(next) | Description |
| --- | --- | --- |
| 0 | Q(prev) | Hold (no change) |
| 1 | Q′(prev) | Toggle (switch) |

## T Flip-Flop is derived

- From JK Flip-Flop

  - If we connect J = K = T, a JK flip-flop behaves like a T flip-flop.

  - Because when J=K=1 → JK FF toggles.

- From D Flip-Flop

  - If we connect input T XOR Q to D, the D flip-flop behaves like T flip-flop.

Types of T Flip-Flops

- Level Triggered T Flip-Flop
Works during the level (high or low) of clock. Not common.

- Edge Triggered T Flip-Flop
Changes state only on the rising (↑) or falling (↓) edge of the clock pulse.

- Master-Slave T Flip-Flop

  - Two flip-flops connected in series (Master + Slave).

  - Eliminates problems like race-around condition.

Race-Around Condition in T Flip-Flop

- Like JK FF, if clock pulse is too long and T=1, the output may toggle multiple times → unstable.

- Solution: Use edge-triggered or Master-Slave T flip-flop.

## Applications of T Flip-Flops

- Binary Counters (up-counters, down-counters).

- Frequency Divider Circuits

- Shift Registers.

- Toggle Switch in Digital Systems.

- Sequence Generator.

- The T Flip-Flop is mainly used for toggling operations.

- It can be implemented from JK flip-flop (J=K=T) or D flip-flop (D = T ⊕ Q).

- Its toggle property makes it very useful in counters and frequency division circuits.

D FLIP-FLOPS

A D Flip-Flop is a sequential logic circuit that stores one bit of data.
It transfers the input (denoted by D, for Data) to the output (Q) on a specific clock edge (rising or falling).

It is called a delay flip-flop because the output is simply the delayed version of the input.

 The D flip-flop has one data input (D) and one clock input (CLK).

 The output Q takes the value of D when the clock pulse occurs (on the rising or falling edge).

 Until the next clock pulse, Q remains constant (stores the value).

## Truth Table

| Clock |  |  | D |  | Q(next) |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ↑ (rising edge) |  |  | 0 |  | 0 |  | Reset state |  |
| ↑ (rising edge) |  |  | 1 |  | 1 |  | Set state |  |
| 0 / ↓ (no edge) |  |  | X |  | Q |  | No change (hold) |  |
| Characteristic equation |  |  |  |  |  |  |  |  |
| Feature | D Flip-Flop | SR Flip-Flop |  | JK Flip-Flop |  | T Flip-Flop |  |
| Inputs | 1 (D) | 2 (S, R) |  | 2 (J, K) |  | 1 (T) |  |
| Invalid state | None | S=R=1 invalid |  | None |  | None |  |
| Operation | Qnext = D | Set/Reset |  | Toggle/Set/Reset |  | Toggle |  |
| Simplicity | Very simple | Moderate |  | Complex |  | Simple |  |