# Registers and Their Types

A register is a small, high-speed storage unit located inside the CPU. It holds data, instructions, or addresses temporarily during execution. Registers are essential for performing arithmetic, logic, and control operations efficiently. They are faster than cache or main memory because they are directly connected to the CPU’s control circuits.

## Key Points:

Operate at CPU speed

Used for temporary storage of data

Directly accessible by the processor

Usually limited in number (8, 16, or 32 registers)

Each register has a fixed bit size (8-bit, 16-bit, 32-bit, etc.)

## Types of Registers

Registers are classified according to their purpose and function. Below are the main types including Shift Registers.

| Register Type | Function |
| --- | --- |
| Accumulator (AC) | Stores intermediate results of arithmetic and logical operations. |
| Program Counter (PC) | Holds the address of the next instruction to be executed. |
| Memory Address Register (MAR) | Stores the address of the memory location to be accessed. |
| Memory Data Register (MDR) | Holds the actual data being transferred to/from memory. |
| Instruction Register (IR) | Stores the instruction currently being decoded and executed. |
| General Purpose Registers (GPRs) | Used for temporary data storage during program execution. |
| Stack Pointer (SP) | Holds the address of the top element of the stack. |
| Status/Flag Register | Contains flag bits that show results of operations performed by the ALU. |
| Temporary Register | Used internally by the CPU to hold intermediate results. |
| Index Register | Used in indexed addressing mode to modify operand addresses. |
| Shift Register | Stores binary data and can shift bits to the left or right upon clock pulses. |

## Shift Registers

A Shift Register is a type of register that stores binary data and can shift the bits (data) to the left or right when triggered by clock pulses. It is built using flip-flops connected in series, where the output of one flip-flop becomes the input of the next.

## Types of Shift Registers:

Serial-In Serial-Out (SISO): Data enters and exits one bit at a time.

Serial-In Parallel-Out (SIPO): Data enters serially and is output in parallel.

Parallel-In Serial-Out (PISO): Data enters in parallel and exits serially.

Parallel-In Parallel-Out (PIPO): Data enters and exits in parallel.

Bi-Directional Shift Register: Shifts data both left and right.

Universal Shift Register: Can perform all shift operations (serial/parallel, left/right).

## Applications of Shift Registers:

Data storage and transfer

Serial-to-parallel and parallel-to-serial conversion

Digital signal processing

Communication systems

Counters and sequence generators

Multiplication/division of binary numbers by shifting

Registers are the fastest and most crucial memory units inside the CPU. They control the flow of data—fetching instructions, storing results, and managing execution. Among them, Shift Registers are special because they can move bits in different directions, making them useful in data communication, conversion, and arithmetic operations.

Serial-In-Serial-Out

The shift registers which allows serial input and produces serial output

Here we use D flip-flop which is cascaded, that means output of one D flipflop is connected to the input of next D flipflop

All flip flops are synchronous to each other

Here, we can send the bits serially from the input to left

most D flipflop that is serial input and we get the output from right most D flipflop that is called serial output

For every positive edge triggering of clock signal, the data is shifted from one stage to next.